# Project Aura: Technical Documentation

## Executive Summary

Project Aura is a localized, high-performance edge-computing infrastructure designed to serve as a decentralized "Digital Brain" for a makerspace. By utilizing a 9U 10-inch rack to house a diverse cluster of ARM and NVIDIA-based nodes, Aura provides a low-latency, privacy-focused alternative to cloud-based AI. The project focuses on distributed inference, local Retrieval-Augmented Generation (RAG), and environment-aware intelligence.

## Infrastructure & Hardware Architecture

### Enclosure & Mounting System

The physical core of the project is the Tecmojo 9U Network Rack, a 10-inch wide mini server rack that provides vertical space for up to nine rack units (U).

- Form Factor: The 10-inch width allows for greater portability than standard 19-inch racks while providing enough internal volume for airflow around the SoC boards.

- Component Density: Rack shelves and custom mounting brackets are used to stack the diverse array of boards, maximizing the 9U vertical capacity.

### Heterogeneous Compute Cluster

Aura utilizes a diverse set of hardware architectures to distribute AI and management tasks across the most efficient processors.

- AI Inference Engine (NVIDIA Jetson Orin Nano Super): This is the primary compute powerhouse, featuring 1024 CUDA cores and 32 Tensor Cores based on the Ampere architecture. It delivers up to 67 TOPS (INT8) of AI performance, making it the central hub for running the project's Large Language Models (LLMs) and real-time vision tasks.

- The Cluster Memory (Orange Pi 3B+): This node serves as the dedicated host for the local Vector Database (e.g., ChromaDB or Qdrant), handling the high-speed data retrieval required for RAG-based assistant responses.

- Management & Orchestration (Raspberry Pi 4): As a versatile 1.8 GHz quad-core ARM node, the Pi 4 acts as the cluster's head node, managing networking traffic and task distribution.

- Sensor & Legacy Interface (Raspberry Pi 3B+): This node provides backward compatibility and handles low-power environmental sensor monitoring via its GPIO pins.

### Networking Backbone

The network is designed as an "internal grid" that remains functional even without an external internet connection.

- Wireless Gateway: The TP-Link AX1500 Wi-Fi 6 Travel Router (TL-WR1502X) is configured in "Client Mode" to bridge existing building Wi-Fi into the internal cluster network. Wi-Fi 6 (802.11ax) ensures low-latency communication with mobile terminals across the makerspace.

- Gigabit Backbone: Central connectivity is provided by the TP-Link TL-SG108 8-Port Gigabit Switch. This unmanaged switch handles the high-bandwidth inter-node communication necessary for cluster-wide task sharing.

- Precision Cabling: 10Gsupxsel 0.5ft Cat 6 cables are used throughout. The ultra-short length is critical for the 10-inch rack to prevent "cable spaghetti" and ensure unobstructed airflow around the SoC heatsinks.

### High-Output Power Distribution

Project Aura implements a centralized power system to eliminate the need for individual wall-wart power supplies.

- Main Distribution: A 10-inch Rack PDU (15A) provides the main AC power rail for the entire rack.

- USB-C GaN Station: A 260W 7-Port Desktop USB-C Charger acts as the primary DC transformer. Its multi-port GaN architecture can simultaneously provide the high-wattage USB Power Delivery (PD) required by the Jetson Orin Nano (up to 25W peak) and the Raspberry Pi 4 (15W peak) without thermal throttling.

### Thermal & Cable Management

Maintaining stable temperatures for the Jetson and Pi nodes is critical for sustained AI inference.

- Airflow Optimization: Components are spaced vertically within the 9U rack to allow hot air to rise and be exhausted from the open structure.

- Cable Routing: The use of short patch cables and Velcro straps maintains clear paths for intake fans, particularly for the Jetson Orin Nano, which can operate in various power profiles (7W to 25W) depending on cooling efficiency.

Intelligence Layer: Local RAG & LLM Orchestration

The Intelligence Layer represents the "Digital Brain" of Project Aura, defining how specialized hardware nodes collaborate to process complex language tasks and retrieve specific knowledge without cloud assistance.

### Distributed Inference Core

The inference strategy leverages the heterogeneous strengths of the cluster to ensure sub-second latency and high throughput for Large Language Models (LLMs).

- Primary Inference Node (NVIDIA Jetson Orin Nano Super):

  - Role: Serves as the high-performance execution engine for 1B–8B parameter models (e.g., Llama 3.2 or Mistral).

  - Implementation: Utilizes Ollama or llama.cpp backends to run 4-bit quantized models, delivering between 28 and 55 tokens per second.

  - GPU Acceleration: Uses the Ampere-based GPU and CUDA cores to perform tensor computations, bypassing the CPU to maintain sub-second response times.

- Task Management (Raspberry Pi 4):

  - Role: Acts as the Inference Gateway, receiving user queries and deciding whether to route them to the Jetson for heavy generation or handle lightweight logic locally.

  - Load Balancing: Can utilize Nginx or a custom FastAPI wrapper to distribute requests across the cluster if multiple inference nodes are added in the future.

### Local Retrieval-Augmented Generation (RAG) Engine

RAG enables Aura to answer questions using private makerspace data by dynamically injecting relevant context into the LLM's prompt at runtime.

- Knowledge Storage (Orange Pi 3B+):

  - Vector Database: Hosts a production-ready instance of Qdrant or ChromaDB. This node is dedicated to indexing "Knowledge Chunks"—mathematical representations of makerspace manuals and safety guides.

  - Search Optimization: Uses HNSW (Hierarchical Navigable Small Worlds) indexing to perform millisecond similarity searches even as the database grows to thousands of documents.

- The RAG Lifecycle:

  - Ingestion: Documents are split into "smart chunks" and converted into vectors (embeddings) using a model like nomic-embed-text.

  - Retrieval: When a query is received, the system finds the top-k most similar segments in the Orange Pi's vector store.

  - Augmentation: The retrieved text is prepended to the user's prompt (e.g., "Using this manual segment: [Retrieved Text], answer the following...").

### Multi-Node Orchestration & Communication

Orchestration transforms a collection of individual boards into a cohesive, collaborative ecosystem.

- Asynchronous Messaging (MQTT):

  - The Raspberry Pi 4 hosts a Mosquitto MQTT Broker, which facilitates ultra-lightweight communication between nodes.

  - The Raspberry Pi 3B+ acts as a "Sensor Watchdog," publishing real-time environmental data (e.g., air quality) to a specific MQTT topic that the LLM can query for context-aware safety responses.

- Agentic Orchestration:

  - The system can be configured with specialized agents (e.g., a "Researcher Agent" for RAG and a "Safety Agent" for sensors) that collaborate to solve multi-step problems.

  - Model Parallelism: Advanced setups can split a single model across devices, though Aura primarily uses "Task Parallelism" where each node manages a discrete segment of the intelligence pipeline (Inference vs. Vector Search) to minimize network overhead.

Implementation Schedule

This 10-week schedule for Project Aura is designed to transform your specific hardware—the NVIDIA Jetson, Raspberry Pi, and Orange Pi cluster—into a functional, high-performance edge AI infrastructure within your Tecmojo 9U rack.

### Phase 1: Physical Infrastructure & Networking (Weeks 1–2)

The first phase focuses on building the "skeleton" of the grid and establishing a robust communication backbone.

- Week 1: Physical Rack Assembly & Power

  - Mount the 10-inch PDU and the 260W USB-C Charger into the lower units of the Tecmojo 9U rack.

  - Install the SoC boards (Jetson Orin Nano, Orange Pi 3B+, Raspberry Pi 4, and 3B+) onto rack shelves using standoffs.

  - Route the USB-C power cables from the 260W charger to each board, ensuring the Jetson Orin Nano is connected to a high-wattage port (65W+ preferred for peak performance).

- Week 2: Networking Gateway & Backbone

  - Configure the TP-Link AX1500 (TL-WR1502X) in "Client Mode" to bridge your building's Wi-Fi to the rack's Ethernet backbone.

  - Connect the AX1500 LAN port to the TP-Link TL-SG108 8-port Switch using a 0.5ft Cat 6 cable.

  - Interconnect all SoC nodes to the switch and assign Static IPs to each device (e.g., .10 for Jetson, .11 for RPi 4, etc.) to ensure reliable cluster orchestration.

### Phase 2: OS Deployment & Node Specialization (Weeks 3–4)

This phase moves from hardware to software, initializing the specialized operating environments for each node.

- Week 3: System Flashing & Base Configuration

  - Flash NVIDIA JetPack 6.x to an NVMe drive or microSD for the Jetson Orin Nano; enable "Max Performance" mode using nvpmodel.

  - Install Raspberry Pi OS (64-bit) on the Pi 4 and 3B+ nodes, and a compatible Linux distro on the Orange Pi 3B+.

  - Verify SSH access across all nodes and synchronize system clocks using NTP.

- Week 4: AI Drivers & Environment Setup

  - Install CUDA and TensorRT on the Jetson Orin Nano to unlock its 67 TOPS of AI potential.

  - Install Ollama on the Jetson (for heavy inference) and the Raspberry Pi 4 (for lightweight task coordination).

### Phase 3: Intelligence & Knowledge Grid (Weeks 5–7)

During these weeks, you will build the "Digital Brain" of Aura by deploying models and the local memory system.

- Week 5: Base LLM Deployment

  - Deploy Llama 3.2 (3B or 8B) or Phi-3 on the Jetson Orin Nano using Ollama.

  - Benchmark performance (tokens per second) to ensure sub-second response times.

- Week 6: Vector Database & Memory Hub

  - Install Qdrant or ChromaDB on the Orange Pi 3B+ to serve as the cluster's "Memory".

  - Initialize the first database collections for makerspace technical manuals.

- Week 7: RAG Pipeline Integration

  - Develop a Python-based Retrieval-Augmented Generation (RAG) script that allows the Jetson to query the Orange Pi's vector store for context before generating a response.

  - Test the system by asking "Aura" specific questions from an uploaded PDF manual.

### Phase 4: Integration & Edge Deployment (Weeks 8–10)

The final phase connects Aura to the physical world and refines the user experience.

- Week 8: Sensor Mesh & MQTT Broker

  - Deploy a Mosquitto MQTT Broker on the Raspberry Pi 4 to act as the cluster's "communication hub".

  - Connect sensors to the Raspberry Pi 3B+ GPIO pins and write a script to publish environmental data (e.g., room temperature) to Aura.

- Week 9: Orchestration & Terminal UI

  - Set up Open WebUI or a custom FastAPI dashboard to provide a user-friendly interface for makerspace students.

  - Configure the system so that "Aura" can proactively warn users if sensors detect unsafe conditions (e.g., poor air quality while using a laser cutter).

- Week 10: Optimization & Final Presentation

  - Perform "Stress Tests" by running multiple queries simultaneously to verify cluster stability.

  - Finalize the project's technical documentation and "User Manual" for students.

| Week | Phase | Key Tasks & Milestones | Primary Hardware Involved |
| --- | --- | --- | --- |
| 1 | Physical Assembly | Mount SoC boards in the Tecmojo 9U rack; install the 15A PDU and 260W USB-C charger. | Tecmojo Rack, PDU, USB-C Charger |
| 2 | Networking | Configure TL-WR1502X Router to Client Mode; link all nodes via TL-SG108 Switch and 0.5ft Cat 6 cables. | TP-Link Router, Switch, Cat 6 Cables |
| 3 | OS Flashing | Flash JetPack 6.x to Jetson; install Raspberry Pi OS (64-bit) and Orange Pi compatible Linux distros. | Jetson Orin, RPi 4, OPi 3B+ |
| 4 | AI Drivers | Enable CUDA and TensorRT on Jetson; verify PCIe settings and GPU performance modes. | NVIDIA Jetson Orin Nano |
| 5 | LLM Deployment | Deploy Llama 3.2 (3B or 8B) or Phi-3 using Ollama on the Jetson Orin Nano. | NVIDIA Jetson Orin Nano |
| 6 | Memory Setup | Initialize ChromaDB or Qdrant on the Orange Pi 3B+; create the first knowledge collections. | Orange Pi 3B+, RPi 4 |
| 7 | RAG Pipeline | Build the Python-based Retrieval-Augmented Generation logic between Jetson and Orange Pi. | Jetson Orin, OPi 3B+ |
| 8 | Sensor Mesh | Connect sensors to RPi 3B+; set up Mosquitto MQTT broker on RPi 4 for data fusion. | RPi 3B+, RPi 4 |
| 9 | User Interface | Launch Open WebUI or a custom FastAPI terminal dashboard for makerspace interaction. | RPi 4, Terminal Nodes |
| 10 | Optimization | Conduct full-load stress tests; finalize project documentation and student "Safety Agent" protocols. | Entire Cluster |